// UART Transmitter

module uart_tx (

input wire clk,                    // System clock (100 MHz)

input wire reset,                  // Active-high reset

input wire tx_start,               // Start transmission signal (pulse)

input wire [7:0] tx_data,          // 8-bit parallel data to transmit

output reg tx_done,                // Transmission complete flag

output reg tx_out                  // Serial output line

);

// Timing parameters

parameter CLK_FREQ = 100_000_000;  // Clock frequency: 100 MHz

parameter BAUD_RATE = 115200;      // Baud rate: 115200 bits/sec

// Calculate number of clock cycles per bit (100MHz / 115200 ≈ 868)

localparam BIT_PERIOD = (CLK_FREQ + BAUD_RATE/2) / BAUD_RATE;

// State encoding using one-hot encoding (3-bit states)

localparam [2:0] IDLE   = 3'b000,  // Waiting for transmission request

START  = 3'b001,  // Sending start bit (0)

DATA   = 3'b010,  // Sending 8 data bits

STOP   = 3'b011;  // Sending stop bit (1)

// Internal registers

reg [2:0] state, next_state;       // Current and next state

reg [7:0] data_reg;                // Holds data during transmission

reg [2:0] bit_index;               // Tracks which data bit is being sent (0-7)

reg [15:0] clk_counter;            // Counts clock cycles within each bit

// ================================================

// STATE REGISTER (SYNCHRONOUS)

// ================================================

// Updates state on every clock edge

always @(posedge clk or posedge reset) begin

if (reset) state <= IDLE;      // Reset to IDLE state

else state <= next_state;      // Normal state transition

end

// ================================================

// NEXT STATE LOGIC (COMBINATIONAL)

// ================================================

// Determines next state based on current state and conditions

always @(*) begin

next_state = state;            // Default: stay in current state

case (state)

IDLE:   if (tx_start) next_state = START;           // Start bit when tx_start pulses

START:  if (clk_counter == BIT_PERIOD - 1) next_state = DATA;  // Move to DATA after start bit duration

DATA:   if ((bit_index == 3'd7) && (clk_counter == BIT_PERIOD - 1)) next_state = STOP;  // Move to STOP after 8 bits

STOP:   if (clk_counter == BIT_PERIOD - 1) next_state = IDLE;  // Return to IDLE after stop bit

default: next_state = IDLE;                         // Safety: always return to IDLE

endcase

end

// ================================================

// DATA REGISTER

// ================================================

// Latch the input data when transmission starts

always @(posedge clk or posedge reset) begin

if (reset) begin

data_reg <= 8'b0;           // Clear on reset

end else if (tx_start && state == IDLE) begin

data_reg <= tx_data;        // Capture data at start of transmission

end

end

// ================================================

// CLOCK COUNTER

// ================================================

// Counts clock cycles for each bit duration

always @(posedge clk or posedge reset) begin

if (reset) begin

clk_counter <= 16'b0;       // Reset counter

end else if (state != IDLE) begin           // Count only during active transmission

if (clk_counter == BIT_PERIOD - 1) begin

clk_counter <= 16'b0;   // Reset at end of bit period

end else begin

clk_counter <= clk_counter + 1;  // Increment counter

end

end else begin

clk_counter <= 16'b0;       // Reset when in IDLE state

end

end

// ================================================

// BIT INDEX COUNTER

// ================================================

// Tracks which data bit (0-7) is being transmitted

always @(posedge clk or posedge reset) begin

if (reset) begin

bit_index <= 3'b0;          // Reset bit index

end else if (state == DATA) begin          // Only increment during DATA state

if (clk_counter == BIT_PERIOD - 1) begin  // At end of current bit

if (bit_index == 3'd7) begin

bit_index <= 3'b0;  // Reset after 8th bit

end else begin

bit_index <= bit_index + 1;  // Move to next bit

end

end

end else begin

bit_index <= 3'b0;          // Reset when not in DATA state

end

end

// ================================================

// TX OUTPUT LOGIC (Serial Output)

// ================================================

// Generates the serial output based on current state

always @(posedge clk or posedge reset) begin

if (reset) begin

tx_out <= 1'b1;             // Default high (idle state)

end else begin

case (state)

IDLE:   tx_out <= 1'b1;            // High when idle

START:  tx_out <= 1'b0;            // Start bit = 0

DATA:   tx_out <= data_reg[bit_index];  // Send LSB first (bit_index 0 = LSB)

STOP:   tx_out <= 1'b1;            // Stop bit = 1

default: tx_out <= 1'b1;           // Default high

endcase

end

end

// ================================================

// TX DONE SIGNAL

// ================================================

// Pulses high for one clock cycle when transmission completes

always @(posedge clk or posedge reset) begin

if (reset) begin

tx_done <= 1'b0;            // Clear done flag

end else begin

// Assert done at the END of stop bit (last clock cycle of STOP state)

tx_done <= (state == STOP) && (clk_counter == BIT_PERIOD - 1);

end

end

endmodule

// ===========================================================================

// UART Receiver - FIXED VERSION

// ===========================================================================

module uart_rx (

input wire clk,                    // System clock (100 MHz)

input wire reset,                  // Active-high reset

input wire rx_in,                  // Serial input line

output reg [7:0] rx_data,          // 8-bit parallel received data

output reg rx_done,                // Reception complete flag (pulse)

output reg rx_error                // Error flag (stop bit not detected)

);

// Timing parameters (must match transmitter)

parameter CLK_FREQ = 100_000_000;  // Clock frequency: 100 MHz

parameter BAUD_RATE = 115200;      // Baud rate: 115200 bits/sec

// Calculate timing constants

localparam BIT_PERIOD = (CLK_FREQ + BAUD_RATE/2) / BAUD_RATE;      // Full bit duration

localparam HALF_BIT_PERIOD = BIT_PERIOD / 2;                       // Half bit (for sampling)

// State encoding (same as transmitter for consistency)

localparam [2:0] IDLE   = 3'b000,  // Waiting for start bit

START  = 3'b001,  // Receiving start bit

DATA   = 3'b010,  // Receiving 8 data bits

STOP   = 3'b011;  // Receiving stop bit

// Internal registers

reg [2:0] state, next_state;       // Current and next state

reg [7:0] data_reg;                // Shift register for incoming bits

reg [2:0] bit_index;               // Tracks which data bit is being received

reg [15:0] clk_counter;            // Counts clock cycles within each bit

reg rx_sync1, rx_sync2;            // Double synchronization registers

reg rx_clean;                      // Clean, synchronized input

// ================================================

// INPUT SYNCHRONIZATION (Metastability Protection)

// ================================================

// Triple synchronizer to avoid metastability from asynchronous rx_in

always @(posedge clk or posedge reset) begin

if (reset) begin

rx_sync1 <= 1'b1;          // Reset to idle state (high)

rx_sync2 <= 1'b1;

rx_clean <= 1'b1;

end else begin

rx_sync1 <= rx_in;         // First stage: async to sync domain

rx_sync2 <= rx_sync1;      // Second stage: reduce metastability

rx_clean <= rx_sync2;      // Third stage: clean signal for logic

end

end

// ================================================

// STATE REGISTER (SYNCHRONOUS)

// ================================================

always @(posedge clk or posedge reset) begin

if (reset) state <= IDLE;      // Reset to IDLE state

else state <= next_state;      // Normal state transition

end

// ================================================

// NEXT STATE LOGIC (COMBINATIONAL)

// ================================================

always @(*) begin

next_state = state;            // Default: stay in current state

case (state)

IDLE:   if (rx_clean == 1'b0) next_state = START;  // Detect falling edge = start bit

START:  if (clk_counter == BIT_PERIOD - 1) next_state = DATA;    // After full start bit

DATA:   if ((bit_index == 3'd7) && (clk_counter == BIT_PERIOD - 1)) next_state = STOP;  // After 8 bits

STOP:   if (clk_counter == BIT_PERIOD - 1) next_state = IDLE;    // After full stop bit

default: next_state = IDLE;                         // Safety: return to IDLE

endcase

end

// ================================================

// CLOCK COUNTER

// ================================================

// Counts clock cycles to time bit boundaries

always @(posedge clk or posedge reset) begin

if (reset) begin

clk_counter <= 16'b0;      // Reset counter

end else if (state != IDLE) begin           // Count only during active reception

if (clk_counter == BIT_PERIOD - 1) begin

clk_counter <= 16'b0;  // Reset at end of bit period

end else begin

clk_counter <= clk_counter + 1;     // Increment counter

end

end else begin

clk_counter <= 16'b0;      // Reset when in IDLE state

end

end

// ================================================

// BIT INDEX COUNTER

// ================================================

// Tracks which data bit (0-7) is being received

always @(posedge clk or posedge reset) begin

if (reset) begin

bit_index <= 3'b0;         // Reset bit index

end else if (state == DATA) begin           // Only increment during DATA state

if (clk_counter == BIT_PERIOD - 1) begin  // At end of current bit

if (bit_index == 3'd7) begin

bit_index <= 3'b0; // Reset after 8th bit

end else begin

bit_index <= bit_index + 1;     // Move to next bit

end

end

end else begin

bit_index <= 3'b0;         // Reset when not in DATA state

end

end

// ================================================

// DATA SAMPLING LOGIC (FIXED)

// ================================================

// Samples data bits in the MIDDLE of each bit period (most stable point)

always @(posedge clk or posedge reset) begin

if (reset) begin

data_reg <= 8'b0;          // Clear shift register

end else if (state == DATA) begin            // Only sample during DATA state

// Sample at the middle of each bit period (most reliable)

if (clk_counter == HALF_BIT_PERIOD - 1) begin

data_reg[bit_index] <= rx_clean;     // Store bit at correct position

end

end

end

// ================================================

// OUTPUT DATA AND DONE SIGNAL

// ================================================

// Presents received data and pulses done signal

always @(posedge clk or posedge reset) begin

if (reset) begin

rx_data <= 8'b0;           // Clear output data

rx_done <= 1'b0;           // Clear done flag

end else if (state == STOP && clk_counter == BIT_PERIOD - 1) begin

rx_data <= data_reg;       // Latch data at END of stop bit

rx_done <= 1'b1;           // Pulse done for one clock cycle

end else begin

rx_done <= 1'b0;           // Clear done signal

end

end

// ================================================

// ERROR DETECTION LOGIC (FIXED)

// ================================================

// Checks if stop bit is HIGH at the middle of stop bit period

always @(posedge clk or posedge reset) begin

if (reset) begin

rx_error <= 1'b0;          // Clear error flag

end else if (state == STOP) begin             // During stop bit reception

// Check at middle of stop bit period (most reliable)

if (clk_counter == HALF_BIT_PERIOD - 1) begin

rx_error <= (rx_clean != 1'b1);      // Error if stop bit is not HIGH

end

end else if (state == IDLE) begin

rx_error <= 1'b0;          // Clear error when back to IDLE

end

end

endmodule

// ===========================================================================

// UART TOP MODULE (Wrapper)

// ===========================================================================

module uart (

input wire clk,                    // System clock (100 MHz)

input wire reset,                  // Active-high reset

// Transmitter interface

input wire tx_start,               // Start transmission signal

input wire [7:0] tx_data,          // 8-bit parallel data to transmit

output wire tx_done,               // Transmission complete flag

output wire tx_out,                // Serial output line

// Receiver interface

input wire rx_in,                  // Serial input line

output wire [7:0] rx_data,         // 8-bit parallel received data

output wire rx_done,               // Reception complete flag

output wire rx_error               // Error flag

);

// Instantiate transmitter module

uart_tx transmitter (

.clk(clk),

.reset(reset),

.tx_start(tx_start),

.tx_data(tx_data),

.tx_done(tx_done),

.tx_out(tx_out)

);

// Instantiate receiver module

uart_rx receiver (

.clk(clk),

.reset(reset),

.rx_in(rx_in),

.rx_data(rx_data),

.rx_done(rx_done),

.rx_error(rx_error)

);

endmodule

### Transmitter (uart_tx):

- FSM States: IDLE → START → DATA → STOP → IDLE

- Bit Timing: Counts BIT_PERIOD (868 cycles) for each bit

- LSB First: Transmits LSB (bit 0) first, MSB (bit 7) last

- Output: Start bit (0) → 8 data bits → Stop bit (1)

### Receiver (uart_rx):

- Metastability Protection: Triple synchronization for rx_in

- Sampling Point: Samples data in the MIDDLE of each bit period

- Error Detection: Checks if stop bit is HIGH

- Data Assembly: Builds byte by sampling individual bits

### Timing Calculations:

- At 100MHz, 115200 baud = 100,000,000 / 115,200 ≈ 868 cycles/bit

- Half period = 868 / 2 = 434 cycles

- Sampling occurs at cycle 433 (middle of each bit)

// ============================================================

// IMPROVED TESTBENCH

// ============================================================

module uart_tb;

// Testbench parameters

parameter CLK_PERIOD = 10;      // 100 MHz clock (10ns period = 100MHz)

parameter REAL_BIT_TIME = 868;  // 115200 baud at 100MHz = 100e6/115200 ≈ 868 cycles

// Testbench signals

reg clk;                        // System clock

reg reset;                      // Active-high reset signal

// Transmitter interface signals

reg tx_start;                   // Pulse to start transmission

reg [7:0] tx_data;              // 8-bit data to transmit

wire tx_done;                   // Flag indicating transmission complete

wire tx_out;                    // Serial output line

// Receiver interface signals

wire [7:0] rx_data;             // 8-bit received data

wire rx_done;                   // Flag indicating reception complete

wire rx_error;                  // Flag indicating reception error

// Test statistics counters

integer test_count;             // Total number of tests executed

integer pass_count;             // Number of passed tests

integer error_count;            // Number of failed tests

// ============================================================

// DEVICE UNDER TEST (DUT) INSTANTIATION

// ============================================================

// Instantiate the complete UART module

// Note: Using loopback - transmitter output connected to receiver input

uart uart_inst (

.clk(clk),                  // Connect testbench clock

.reset(reset),              // Connect testbench reset

// Transmitter connections

.tx_start(tx_start),        // Connect transmit start signal

.tx_data(tx_data),          // Connect transmit data

.tx_done(tx_done),          // Monitor transmit done flag

.tx_out(tx_out),            // Monitor serial output

// Receiver connections with LOOPBACK

.rx_in(tx_out),             // LOOPBACK: Connect TX output to RX input

.rx_data(rx_data),          // Monitor received data

.rx_done(rx_done),          // Monitor receive done flag

.rx_error(rx_error)         // Monitor receive error flag

);

// ============================================================

// CLOCK GENERATION

// ============================================================

// Generate 100MHz clock (10ns period: 5ns high, 5ns low)

initial begin

clk = 0;                    // Initialize clock to 0

forever #(CLK_PERIOD/2) clk = ~clk;  // Toggle every 5ns

end

// ============================================================

// WAVEFORM DUMP (for debugging with GTKWave)

// ============================================================

initial begin

$dumpfile("uart_tb.vcd");   // Create VCD file for waveform viewing

$dumpvars(0, uart_tb);      // Dump ALL variables in testbench hierarchy

#5000000;                   // Stop dump after 5ms (prevents huge files)

$dumpoff;                   // Turn off dumping

end

// ============================================================

// MAIN TEST SEQUENCE

// ============================================================

initial begin

// Initialize all signals

reset = 1;                  // Assert reset

tx_start = 0;               // No transmission requested

tx_data = 0;                // Clear transmit data

test_count = 0;             // Reset test counter

pass_count = 0;             // Reset pass counter

error_count = 0;            // Reset error counter

// Generate reset pulse (2 clock cycles)

#(CLK_PERIOD * 2);          // Wait 20ns

reset = 0;                  // Deassert reset

#(CLK_PERIOD * 2);          // Wait 20ns for system to stabilize

// Display test header

$display("\n=====================================");

$display(" UART TEST SUITE STARTED");

$display("=====================================");

// Execute test cases with various data patterns

// Test 1: All zeros - tests LSB and MSB at 0

run_test(8'h00, "All zeros");

// Test 2: All ones - tests LSB and MSB at 1

run_test(8'hFF, "All ones");

// Test 3: Alternating pattern 01010101

run_test(8'h55, "01010101");

// Test 4: Alternating pattern 10101010

run_test(8'hAA, "10101010");

// Test 5: Clear transition pattern 11110000

run_test(8'hF0, "11110000");

// Test 6: Clear transition pattern 00001111

run_test(8'h0F, "00001111");

// Test 7: Random pattern 10100101

run_test(8'hA5, "10100101");

// Test 8: Complementary pattern 01011010

run_test(8'h5A, "01011010");

// Test 9: Edge case - only MSB set

run_test(8'h81, "10000001");

// Test 10: Edge case - only LSB clear

run_test(8'h7E, "01111110");

// Display final test summary

$display("\n=====================================");

$display(" FINAL SUMMARY");

$display("=====================================");

$display("Total Tests : %0d", test_count);      // Show total tests

$display("Passed      : %0d", pass_count);      // Show passed tests

$display("Failed      : %0d", error_count);     // Show failed tests

if (test_count > 0) begin                       // Avoid division by zero

$display("Success     : %0.1f %%", (pass_count*100.0)/test_count);

end

$display("=====================================");

// Final delay and simulation termination

#(CLK_PERIOD * 100);        // Wait 100 clock cycles

$finish;                    // End simulation

end

// ============================================================

// TEST TASK: run_test

// ============================================================

// Task to execute a single UART transmission/reception test

// Inputs: tdata - 8-bit test data, desc - test description

task run_test;

input [7:0] tdata;          // Test data to transmit

input [80:0] desc;          // Text description of test

begin

test_count = test_count + 1;  // Increment test counter

// Display test information

$display("\nTest %0d : %s", test_count, desc);

$display(" TX Sending : %02X (binary: %08b)", tdata, tdata);

// Synchronize to clock edge for clean timing

@(posedge clk);                 // Wait for rising clock edge

tx_data = tdata;                // Load test data

tx_start = 1;                   // Assert start signal

@(posedge clk);                 // Wait one clock cycle

tx_start = 0;                   // Deassert start signal (one-shot)

// Wait for transmission to complete

wait(tx_done);                  // Block until TX done flag asserted

#(CLK_PERIOD * 2);              // Wait 2 clock cycles for settling

// Wait for reception to complete (loopback)

wait(rx_done);                  // Block until RX done flag asserted

#(CLK_PERIOD * 2);              // Wait 2 clock cycles for settling

// Verify test results

if (rx_data == tdata && rx_error == 0) begin

pass_count = pass_count + 1;        // Increment pass counter

$display(" PASS : RX Received %02X", rx_data);

end else begin

error_count = error_count + 1;      // Increment error counter

$display(" FAIL : Expected %02X, Got %02X, Error=%b",

tdata, rx_data, rx_error);

end

// Inter-test delay (allows UART to return to idle)

// 12 bit times = start + 8 data + stop + 3 idle bits

#(REAL_BIT_TIME * 12);

end

endtask

// ============================================================

// MONITOR PROCESSES (for debugging)

// ============================================================

// Monitor: Print when transmission completes

always @(posedge tx_done) begin

$display("[%0t] TX DONE", $time);  // Display timestamp

end

// Monitor: Print when reception completes

always @(posedge rx_done) begin

// Display timestamp and received data

$display("[%0t] RX DONE: data=%02X", $time, rx_data);

end

// Monitor: Print when reception error occurs

always @(posedge rx_error) begin

$display("[%0t] RX ERROR detected!", $time);  // Warning message

end

endmodule

### 1. Testbench Parameters:

- CLK_PERIOD = 10: Defines 100MHz clock (10ns period)

- REAL_BIT_TIME = 868: Actual clock cycles per bit at 115200 baud

### 2. DUT Instantiation with Loopback:

- Connects tx_out to rx_in for loopback testing

- Allows self-test without external connections

### 3. Clock Generation:

- Generates precise 100MHz clock with 50% duty cycle

- Runs continuously throughout simulation

### 4. Main Test Sequence:

- Reset Phase: 2-cycle reset pulse initialization

- Test Pattern Selection: 10 diverse patterns covering:

  - All zeros/ones (boundary cases)

  - Alternating patterns (01010101, 10101010)

  - Clear transitions (11110000, 00001111)

  - Random and complementary patterns

  - Edge cases (MSB only, LSB only)

### run_test Task Details:

- Synchronization: Uses @(posedge clk) for clean timing

- One-shot Control: tx_start pulses for exactly 1 clock cycle

- Event Waiting: Uses wait() for TX/RX completion signals

- Result Verification: Compares transmitted vs received data

- Inter-test Delay: Waits 12 bit times between tests

### Monitor Processes:

- Real-time display of key events during simulation

- Helps debug timing issues

- Shows exact simulation time of events

### Test Statistics:

- Tracks total, passed, and failed tests

- Calculates success percentage

- Provides clear summary at simulation end

TCL OUTPUT:

=====================================
 UART TEST SUITE STARTED
=====================================

Test 1 :All zeros
 TX Sending : 00 (binary: 00000000)
[86855] TX DONE
[86905] RX DONE: data=00
 PASS : RX Received 00

Test 2 :All ones
 TX Sending : ff (binary: 11111111)
[184155] TX DONE
[184205] RX DONE: data=ff
 PASS : RX Received ff

Test 3 :01010101
 TX Sending : 55 (binary: 01010101)
[281455] TX DONE
[281505] RX DONE: data=55
 PASS : RX Received 55

Test 4 :10101010
 TX Sending : aa (binary: 10101010)
[378755] TX DONE
[378805] RX DONE: data=aa
 PASS : RX Received aa

Test 5 :11110000
 TX Sending : f0 (binary: 11110000)
[476055] TX DONE
[476105] RX DONE: data=f0
 PASS : RX Received f0

Test 6 :00001111
 TX Sending : 0f (binary: 00001111)
[573355] TX DONE
[573405] RX DONE: data=0f
 PASS : RX Received 0f

Test 7 :10100101
 TX Sending : a5 (binary: 10100101)
[670655] TX DONE
[670705] RX DONE: data=a5
 PASS : RX Received a5

Test 8 :01011010
 TX Sending : 5a (binary: 01011010)
[767955] TX DONE
[768005] RX DONE: data=5a
 PASS : RX Received 5a

Test 9 :10000001
 TX Sending : 81 (binary: 10000001)
[865255] TX DONE
[865305] RX DONE: data=81
 PASS : RX Received 81

Test 10 : 01111110
 TX Sending : 7e (binary: 01111110)
[962555] TX DONE
[962605] RX DONE: data=7e
 PASS : RX Received 7e

=====================================
 FINAL SUMMARY
=====================================
Total Tests : 10
Passed      : 10
Failed      : 0
Success     : 100.0 %
=====================================